Reaction Modeling for Chemical Kinetics

Reaction Modeling for Chemical Kinetics

ReactLabTM

J

plus consulting

multivariate analytical technologies

KINSIM

User Manual

© Copyright Jplus Consulting Pty Ltd

### ReactLab™   KINSIM

Reaction Modelling for Chemical Kinetics

USER MANUAL

Contents

ReactLab™   KINSIM	2

ABOUT THIS MANUAL	4

SYSTEM REQUIREMENTS AND INSTALLATION	4

CONTACT INFORMATION AND SUPPORT	4

PART 1: REFERENCE GUIDE	5

INTRODUCTION	5

EXCEL TEMPLATES	5

STARTING THE MATLAB PROGRAM	5

PROGRAM OPERATION, A QUICK EXAMPLE	6

Load Excel File	6

Close Excel File	7

PART 1: REFERENCE GUIDE	8

Introduction	8

The Example for Part 1: the complexation reaction between cyclam and Cu2+ in aqueous solution (Cu_cyclam.xlsx)	8

Model entry and compilation	9

Compile Model	10

Initial Concentrations	12

Rate and equilibrium constant entry	12

The time vector	13

Simulate	13

Plots in Excel	14

Activity-based simulations	16

Activity-based computations	17

Law of Mass Action in Kinetics	18

Approximations of activity coefficients	20

Note on activity coefficient approximations	23

Testing the results	23

Numerical and other options	25

KW	25

Handling protonation equilibria, [H], [OH] and Kw	25

Model entry syntax rules	26

PART 2: EXAMPLES	27

EXAMPLE 2: ENZYME KINETICS (Enzyme_1.xlsx)	27

EXAMPLE 3: ENZYME KINETICS REVISITED (Enzyme_2.xlsx)	29

EXAMPLE 4: ENZYME KINETICS REVISITED AGAIN (Enzyme_3.xlsx)	31

EXAMPLE 5: THE MICHAELIS-MENTEN ANALYSIS (MichaelisMenten.xlsx)	32

EXAMPLE 6: LOTKA-VOLTERRA / PREDATOR-PREY ‘REACTION’ (LotkaVolterra.xlsx)	35

EXAMPLE 7: THE BELOUSOV-ZHABOTINSKY REACTION (BelousovZhabotinsky.xlsx)	37

EXAMPLE 8: THE REACTION OF Fe3+ WITH THIOCYANATE, SCN-, (Fe_SCN.xlsm)	39

EXAMPLE 9: THE REACTION OF Fe3+ WITH 3 LIGANDS L, LARGE CHANGES IN IONIC STRENGTH (Fe_L3.xlsm)	46

References	49

## ABOUT THIS MANUAL

This manual is in two parts. Part 1 comprises a comprehensive description of the program architecture and functionality and provides a systematic guide to using it. Part 2 consists of a series of examples demonstrating specific aspects of simulation in pre-prepared workbooks. All these are provided in the Excel examples folder in the application installation package. Note, when they are installed in the default program files directory, they are automatically assigned read-only status but can of course be copied or re-saved to a suitable working directory.

## SYSTEM REQUIREMENTS AND INSTALLATION

Please refer to the ‘System Requirements and Installation Guide’ available on our website and included with the ReactLab™ download.

## CONTACT INFORMATION AND SUPPORT

support@jplusconsulting.com

www.jplusconsulting.com

Jplus Consulting Pty Ltd
8 Rockdale Close
Killaben Bay NSW 2283
Australia

ABN: 83-135-664-603

# PART 1: REFERENCE GUIDE

## INTRODUCTION

ReactLab™ KINSIM offers extensive modelling and simulation capabilities for chemical reaction kinetics.

The program, including all algorithms and the GUI frontend, has been developed in Matlab and compiled to produce the final deployable application. It requires the free Matlab Component Runtime (MCR) to be installed on the same computer. This allows the program to run on computers without standard Matlab installed and is supplied as part of the installation package.

Model entry and computational output are organised in Excel Workbooks, which are launched from, and dynamically linked to, the ReactLab™ application. Similarly, all numerical analysis options are organised in the Excel format. It is therefore necessary for Excel to be installed on the same computer as ReactLab™.

These results can be further analysed, graphed, or reviewed, just using Excel.

The program requires Excel analysis workbooks to retain a strict format, as is provided in the examples and templates.

A selection of example workbooks accompanies the program in an Excel template subfolder; they are described in detail in Part 2 of this document.

## EXCEL TEMPLATES

To inspect an Excel ReactLab™ KINSIM workbook load it directly into Excel or via the ‘Load Excel’ button in the ReactLab™ GUI.

The workbook is pre-formatted containing three worksheets, the ‘Main’ sheet provides the purpose designed model and parameter entry interfaces. The ‘Activity’ sheet is the output location for activity and ionic strength data. The ‘About’ sheet provides contact information and workbook format version.

## STARTING THE MATLAB PROGRAM

When ReactLab™ KINSIM is launched it also requires the Matlab MCR to initialise. This can take a while and is very much dependant on computer performance, after which the ReactLab™ GUI appears as in Figure 1.

Figure 1: Matlab GUI control panel at startup

This GUI provides the main control interface for the program with a series of pushbuttons on the right-hand side for key functions. These provide all the ReactLab™ program commands and their operations are described over the following pages.

Note depending on the workbook status, some of these functions may be disabled.

## PROGRAM OPERATION, A QUICK EXAMPLE

## Load Excel File

Press Load Excel File in the ReactLab™ GUI to select and link to an Excel Workbook. This will launch a new instance of Excel and load the requested workbook independent of any other open Excel workbooks. Only the ReactLab™ KINSIM workbook template or already developed spreadsheets can be loaded.

Figure 2 displays the developed worksheet (A+B=AB.xlsx) which models the reversible equilibrium reaction between A and B to form AB

ReactLab KINSIM calculates the concentration profiles for the predefined time vector and stores them in the columns I onwards; they can be plotted in Excel in the usual way.

This is a very simple example. To calculate the concentration profiles of A, B and AB all that is required is

- The definition of the reaction model (see equation above):

- The definition of the time vector 
(0, 0.1, 0.2, …, 1.9, 2):

- And the initial concentrations of all species 
([A]0=0.1M, [B]0=0.3M, [AB]0=0M):

An overview of the ‘Main’ worksheet is given in Figure 2. The screen shot was taken after ‘Compile’ and ‘Simulate’ were clicked.

The resulting concentration profiles for all species are stored in the columns down from C43:E43, and are also displayed graphically in the plot. After 2 seconds reaction time the equilibrium is almost reached, the ratio of the concentrations [AB]/([A][B]) = 9.978, which is close to the equilibrium with a value of k+/k- = 10.

Figure 2: A partial view of the ‘Main’ sheet of the ReactLabTM KINSIM file, A+B=AB.xlsx

Important: Excel is launched as an ActiveX server in an independent process linked to ReactLab™ which communicates with it through its Microsoft component object model (COM) interface. The launch of Excel and ActiveX server linkage is initiated by the Matlab program. Linking to an already open workbook in Excel is NOT supported. If a workbook is prepared for analysis following the steps above, it should first be saved and re-opened from the running ReactLab™ application. Note all Excel functionality is available to manipulate the workbook as normal while it is linked to ReactLab™.

## Close Excel File

This will, on confirmation, close the current workbook and link, prior to opening another. If any changes have been made to the workbook the user must first switch focus to the workbook where he/she will be prompted as to whether the changes to the workbook need to be saved.

# PART 1: REFERENCE GUIDE

## Introduction

The procedure for a successful kinetic simulation encompasses several elements, they include:

- the model with all rate (and possibly equilibrium) constants

- the initial concentration for all reactive species

- the required time vector

- if activity-based simulation is performed, additional information about charges of all species and the concentrations of all spectator ions is required

ReactLabTM KINSIM facilitates implementation of the above points.

This manual explains the usage of the software, its user interface based on Excel spreadsheets, the processes of importing data into the spreadsheet, the definition of the chemical model that is used to quantitatively explain the measurements and the process of fitting the parameters.

Note, ReactLabTM KINSIM has the unique property of allowing the simulation to be based on species activities rather than concentrations and thus directly follows the law of mass action which is based on activities. This has several advantages, e.g. the addition of inert salts to approximately maintain ionic strength is no longer required. The complication is that the continued computation of the overall ionic strength requires knowledge of the concentrations of all ionic species, including the spectator ions that do not take part in the equilibria under investigation and this requires additional data input. See Approximations of activity coefficients.

## The Example for Part 1: the complexation reaction between cyclam and Cu2+ in aqueous solution (Cu_cyclam.xlsx)

In order to demonstrate all (or at least most) properties of ReactLabTM KINSIM we need to simulate the kinetics of a reasonably complex chemical process.

The chemical reaction we use as an example in this first section of the manual is the reaction of Cu2+ with the macrocycle cyclam in aqueous solution (see Analysis of reactions in aqueous solution at non-constant pH: No more buffers?; Marcel Maeder, Yorck-Michael Neuhold, Graeme Puxty, Peter King; Phys. Chem. Chem. Phys., 2003, 5, 2836-2841)

The tetradentate ligand cyclam, L, features 4 protonation equilibria to form L, LH, … LH4. Under the conditions of the experiment (pH), only the mono- and di-protonated forms, LH and LH2 react with the copper ion. Further, under these conditions the reactions are irreversible.

The complete reaction scheme is (note we omit charges to improve the readability of the reaction equations):

We have chosen this example as it can be used to demonstrate several important and unique capabilities of ReactLabTM KINSIM:

- it allows the combination of fast (instantaneous) protonation equilibria with slow reaction steps. At the time scale of the experiment (several seconds) the proton exchange is very fast, and the protonation reactions are always and completely in equilibrium. We indicate such equilibria by a double headed arrow () while reactions are represented by simple arrows ().

- It is a reasonably complex reaction mechanism which is supported by the ReactLabTM model entry feature.

- ReactLabTM KINSIM allows the simulations based on species activities rather than concentrations and thus follows the law of mass action directly.

The system comprises 8 species and 3 components, refer to Initial Concentrations, page 12, for more information on these expressions.

## Model entry and compilation

Before performing a simulation, it is necessary to enter a reaction model and to compile it. All model information is placed in the Excel Workbook in the ‘Main’ worksheet. Before proceeding to simulate new data, we start with the pre-formatted template, see Figure 3.

Figure 3: The ReactLabTM KINSIM template, ReactLab_KINSIM_Template.xlsx

The reaction scheme is entered in the model entry area of the ‘Main’ worksheet, see Figure 4.

Figure 4: Entering the Cu/cyclam model, the representation of the reaction model in Excel

Several notes are necessary:

- each reaction step is entered individually, e.g. L+H  LH, which allows for a one-to-one correspondence of each reaction step to its parameter value in the ‘Parameter’ column.

- ReactLab uses the ‘=’ sign to indicate instantaneous equilibria and the ‘>’ for kinetically observable reactions

- See ‘Model entry syntax rules’ for a complete list of syntax rules

- Rate constants are given as their values, while equilibrium constants are always given as their log value

- The Labels are not required, they can be used to facilitate the comprehension

## Compile Model

When model entry is complete press the Compile Model button.

Figure 5: The compile button translates the model into the internal notation required for further computations

ReactLab™ reads in the model and translates it into an internal coefficient form necessary for its subsequent numerical calculations of the concentration profiles of all participating species. This translation distinguishes between equilibrium species and components and kinetic species. In the example the kinetic only species are Cu and CuL, the equilibrium components are L and H and the equilibrium species are L, LH, LH2, LH3, LH4, H and OH. More information in Definition of initial concentrations, page 40.

Compilation adds several entries into the ‘Main’ spreadsheet:

Figure 6: Headings for the initial concentrations for the kinetic species and the equilibrium component concentrations

Figure 7: Headings for all species concentrations

Figure 8: Headings for the definition of charges (and additional information relevant for the calculation of activities)

Species headings are also written to the ‘Activities’ worksheet for subsequent data output (activity coefficients and species activities).

Figure 9: Headings for the outputs in the ‘Activities’ worksheet

## Initial Concentrations

After compilation, the initial conditions/concentrations can be entered into the prepared cells:

Figure 10: Entries of initial concentrations

The reaction solution at time zero contains 0.00394 M CuCl2, 0.00433 M cyclam and 0.00960 M HCl. Note, there is a small excess of cyclam with respect to the metal  and some 2.2 protons per molecule of cyclam to ensure most of the ligand is doubly protonated, LH2. The same initial concentrations could be achieved by e.g. using 0.00433 M doubly protonated cyclam LH2 as the chloride salt with  0.00960-2*0.00433 = 0.00094 M HCl added. The equilibrium concentrations of all equilibrium species are completely defined by the equilibrium components total concentrations.

## Rate and equilibrium constant entry

Equilibrium constants are always entered as their logarithmic (base 10) value. They also always are entered as associations with the associated product on the right. The kinetic rate constants are given as the straight value

In all cases a label for each parameter can be provided. This allows easier reference to mechanisms with multiple steps. These labels and the parameter values themselves are not required prior to compilation.

Note for equilibrium fitting the model can be expressed either in terms of association constants (K’s) as above in Figure 4, or in the alternative format of formation constants (β’s). Using the latter format and the same equilibrium constants, the model would appear as in Figure 11.

Figure 11: The Cu/cyclam model expressed in the formation constant format

The two approaches are essentially identical but yield distinct parameters, though these can be inter-converted quite easily. In this example:

LogβLH = logKLH

logβenH2 = logKLH + logKLH2

This relationship can be extrapolated for higher stoichiometries:

logβn = logK1 + logK2 ...+ logKn

In both cases the equilibrium parameter values are always expressed as a log value which can be positive or negative or indeed zero. This simply means the true parameter value (i.e. anti-logged) is less than or greater than 1 which is permissible for both types of constant.

A discussion of these alternative representations can be found in Maeder and Neuhold (see References).

The key point is that either format will be interpreted correctly by ReactLab™.

Important: It is necessary for any new entry in the Excel workbook to be properly completed, i.e. by hitting return or pressing the arrow key to take the focus away from the cell in question once the desired value has been assigned, otherwise the ‘Incomplete worksheet entry’ warning will be raised. Failing to enter values properly prevents ReactLab™ from accessing the worksheet cell through the ActiveX interface.

## The time vector

Simulation computes the concentrations of all species as a function of time and reports those concentrations at the time points defined in the time vector. ReactLabTM KINSIM offers three methods to define this time vector. ‘linear’, ‘log’ and ‘manual’.

In the ‘linear’ mode npoints are computed between the limits Tmin and Tmax. For the present example this results in the time vector: 0, 600, 1200, 1800, 2400, …. seconds.

In the ‘log’ mode, an exponentially growing set of time points are computed. This mode is useful if there are very fast and very slow processes occurring in the simulated reaction.

In the ‘manual’ mode any number of steadily increasing time points can be defined.

The resulting reaction times are listed in column B of the ‘Main’ worksheet.

## Simulate

After definition of the model, the initial concentrations, the Simulation button initialises the computation of the concentration profiles which are written into the appropriate columns of the spreadsheet.

Figure 12: The computed species concentrations are stored in the appropriate columns

Certain key values are calculated automatically by Excel and are required by ReactLab™, namely the number of times at which the concentrations are reported (n_times), the number of individual chemical species in the mechanism (n_species) and the number of reactions specified in the model for which there is an equilibrium constant field (n_rxns). Do not overwrite the formulae in these cells.

Figure 13: Model information automatically generated by Excel

## Plots in Excel

There are several advantages of using Excel as the user interface; a significant one is the power of Excel to produce graphical output. For all those options, we refer the reader to the Excel manual help and many textbooks. For our purposes the plot of the computed concentration profiles is relatively simple. A standard scatter plot is produced by selecting the relevant section of the spreadsheet:

Figure 14: Selecting the relevant section of the spreadsheet

Followed by selecting the appropriate scatter plot:

Figure 15: Selecting the appropriate Scatter plot

Results in the standard Excel plot:

Figure 16: The resulting basic Excel Scatter Plot

Excel allows any number of annotations and formatting changes, e.g. colour schemes to facilitate the identification of the profiles or changes in the axis labelling for easier reading.

Figure 17: possible annotation of the concentration profiles: the ligand species in red, the metal species in blue and the proton in black.

A few observations might be in place:

- The  decreases during the reaction due to the release of protons (black line) upon the formation of the complex

- Cu and LH2 disappear almost parallel to form ML

- LH3 and LH4 coexist in small concentrations (red lines)

- L and LH2 exist only in very small concentrations at the pH values of the solution (not shown in plot)

## Activity-based simulations

ReactLab™ KINSIM incorporates the facility to simulate chemical kinetics based on species activities rather than concentrations. The thermodynamically relevant rate and equilibrium constants at zero ionic strength are used in this mode. The simulations so far have been performed in the ‘Concentration’ mode. In this section we use the ‘Davies’ equation to approximate the activity coefficients of all charged species, see Approximations of activity coefficients on page 20 for all possible activity approximations available in ReactLabTM.

Figure 18: Activity-based simulation is activated by choosing one of the activity coefficient approximations

For the approximation of activity coefficients, the ionic strength of the solution needs to be computed and to enable that the charges of all reaction species as well as the concentrations of all spectator ions need to be known.

Figure 19: Definition of the charges of all species

Figure 20: Definition of all spectator ion concentrations.

The definition of the spectator ion concentrations is most important. ReactLab allows the definition of the concentration of 1+, 2+, 3+, 1-, 2- and 3- ions. In the example the solution contains 0.00394 M CuCl2 and 0.00960 M HCl, thus the [Cl-] = 2 × [CuCl2] + [HCl]. The entry for (-) anion concentration is ‘=2*I16+J19’ referring to the concentrations defined in the appropriate section.

Figure 21: Concentration profiles for activity-based simulation using the Davies approximation

Compare Figure 21 with Figure 17, the difference is astounding!

While the ionic strength is quite low (between 0.026 and 0.028) the activity coefficients for the higher charged species deviate substantially from 1; they are around 0.85 for the singly charges species, around 0.5 for the doubly charges species, around 0.2 for the triply charges species and around 0.02 for the 4+ species.

Figure 22: Activity coefficient and species activities as a function of the reaction time

## Activity-based computations

The law of mass action is based on species activities, not concentrations. Thus, activity-based simulation is clearly superior and closer to the physio-chemical principles.

## Law of Mass Action in Kinetics

First the principle is demonstrated for the reversible reaction

Kinetic theory defines the following differential equations:

We use the { } brackets to indicate species activities, the standard [ ] brackets are used for concentrations.

In the ‘concentration’ Mode all activity coefficients are set to a value of one and thus the activities are directly replaced by concentrations. In the other Modes, different approximations for the activity coefficients are used.

#### Law of Mass Action in Equilibria

The situation is a bit more complicated for equilibria. We illustrate the principle for equilibria in the following Scheme, using the simple equilibrium A+B=C. In any solution containing the components A and B, after equilibration the activities {A}, {B} and {C} are such that the activities, multiplied in the correct way result in the equilibrium constant

The relationship between concentration, e.g. [A], and activity {A} is given in the Scheme, A is the activity coefficient for A.

The relationship between concentration and activity is easy on paper, see above, but difficult in reality, as the determination of the activity coefficients is usually very challenging. For this reason, in traditional analyses in the law of mass action, the activities are replaced by the species concentrations

While this relationship is not correct, these equations are used because it is too difficult to determine species activities. ReactLab™ KINSIM allows the user to choose between the two implementations of the law of mass action. The mode ‘concentration’ is used of concentrations are to be calculated only, essentially defining all activity coefficients to be one. In the modes ‘Debye-Hückel’, ‘EDH’, ‘Davies’, ‘SIT’ estimates for the activity coefficients are used to approximate species activities, the details of these modes will be explained in a moment.

In any equilibrium analysis, the species concentrations are computed in such a way that the species concentrations or the species activities multiply correctly to result in the appropriate equilibrium constant while simultaneously the re-calculated total concentrations add up to the correct known values. The law of mass action applies to activities, the law of mass conservation applies to concentrations! Figure 23 shows that on the surface the difference is small, however the computations are significantly more complex for the activity calculations.

Concentration

DH, EDH, Davies and SIT

Figure 23: The difference between concentration and activity-based analysis

Activity based simulations require the knowledge of activity coefficients which can be tremendously difficult to determine. ReactLab™ KINSIM uses approximations for the activity coefficients based on Debye-Hückel and further developed, but in essence, similar equations. These approximations are clearly not perfect, but they are better than nothing or better than assuming all activity coefficients are one.

The algorithm used for these computations is a specifically written Newton-Raphson algorithm, we refer to the specialised literature for more information. While the algorithm is standard for the concentration-based analysis, the adaptions for the activity-based simulations are substantial and the computation times accordingly may be slower.

## Approximations of activity coefficients

Box 1 Modes for activity coefficient approximations

Note: For Modes 2-5 the species charges as well as the ionic strength are required and for Mode 3 the species radii are also needed, for Mode 5 specific ion interaction parameters bxk and ion concentrations

The approximations for the activity coefficients which quantitatively describe the relationship between concentration and activity of a species are all based on the work of Debye and Hückel. ReactLab™ KINSIM has incorporated the following approximations: ‘concentration’ no activity correction, the original ‘Debye-Hueckel’ (DH) equation, ‘EDH’ the extended Debye-Hückel, the ‘Davies’ equation as well as the SIT approximation ‘SIT’.

For the Debye-Hückel equation, the activity coefficient of an ion X, X, is approximated as a function of the parameters A, the ionic strength µ and the charge zX of X; the extended Debye-Hückel equation has the additional parameters B and the ionic size rX; the Davies equation has the additional empirical parameter C. The values for A and B are a function of the dielectric constant of the solvent. At 25 ͦC A=0.509 (dm3/mol)1/2 and B=3.286x10-3 (dm3/mol)1/2 (pm)-1 [Manov, G.G., Bates, R.G., Hamer, W.J., Acree, S.F.: Values of the constants in the Debye-Hückel equation for activity coefficients. Journal of American Chemical Society 65, 1765 (1943).]. For C, a value of 0.3 is commonly used.

The equation for the calculation of the total ionic strength, I, is:

Where ci is the concentration of the species i and zi is its charge. The summation is over all ions, including spectator ions that do not take part in the reaction.

All these relevant values have to be defined prior to the computations, the template spreadsheet has default values for aqueous solutions at 25  ͦC. All values can be changed by the user as required.

Figure 24: The definition of the relevant values for the activity approximations in the ‘Main’ worksheet

All activity coefficients strongly depend on the ionic strength, I, of the solution. Therefore, all information required for the computation of the total ionic strength has the be supplied to ReactLab™ KINSIM. This includes the charges of all species, and if the extended Debye-Hückel approximation is used (‘EDH’), then the size of the species in picometers (pm) also need to be defined, see Figure 25.

Figure 25: The definition of the charges, size and SIT parameters for the species in the ‘Main’ worksheet.

The table below provides approximations for ionic radii for common ions, it can be used as a guide for approximations for similar ions. The ionic radii for H+, Cu2+ and OH- are taken from that list, the rest is based on educated guesses.

| charge | Ion / inorganic | size [pm] | Ion / organic | size [pm] |
| --- | --- | --- | --- | --- |
| +/- 1 | H+ | 900 | HCOO-, H2citrate-, CH3NH3+, (CH3)2NH2+ | 350 |
|  | Li+ | 600 | (CH3)3NH+, NH3+CH2COOH, (C2H5)NH3+ | 400 |
|  | Rb+, Cs+,NH4+, Ag+ | 250 | CH3COO-, CH2ClCOO-, (CH3)4N+, C3H7NH3+, (C2H5)2NH2+, NH2CH2COO- | 450 |
|  | K+, Cl-, Br-, I-, CN-, NO2-, NO3- | 300 | CHCl2COO-, CCl3COO-, (C2H5)3NH+, (C3H7)NH3+ | 500 |
|  | OH-, F-, NCS-, NCO-, HS-, ClO3-, ClO4-, BrO3-, IO4-, MnO4- | 350 | C6H5COO-, C6H4OHCOO-, C6H4ClCOO-, C6H5CH2COO-, CH2=CHCH2COO-, (CH3)2C=CHCOO-, (C2H5)4N+, (C3H7)2NH2+ | 600 |
|  | Na+, ClO2-, IO3-, HCO3-, HSO3-, H2PO4-, H2AsO4-, Co(NH3)4(NO2)2+ | 450 | [OC6H2(NO3)3]-, (C3H7)3NH+, CH3OC6H4COO- | 700 |
|  |  |  | (C6H5)2CHCOO-, (C3H7)4N+ | 800 |
| +/- 2 | Hg22+, SO42-, S2O32-, CrO42-, HPO42- | 400 | (COO)22-, Hcitrate2- | 450 |
|  | Pb2+, CO32-, SO32-, [Fe(CN)5NO]2-, [Co(NH3)5Cl]2+, | 450 | H2C(COO)22-, (CH2COO)22-, (CHOHCOO)22- | 500 |
|  | Sr2+, Ba2+, Cd2+, Hg2+, S2-, S2O42-, WO42-, | 500 | C6H4(COO)22-, H2C(CH2COO)22-, (CH2CH2COO)22- | 600 |
|  | Ca2+, Cu2+, Zn2+, Sn2+, Mn2+, Fe2+, Ni2+, Co2+ | 600 | [OOC(CH2)5COO]2-, [OOC(CH2)6COO]2-, [congo red]2- | 700 |
|  | Mg2+, Be2+, | 800 |  | 800 |
| +/- 3 | PO43-, [Fe(CN)6]3-, [Co(NH3)6]3+, [Cr(NH3)6]3+, [Co(NH3)5H2O]3+, | 400 | citrate3- | 500 |
|  | [Co(en)3]3+ | 600 |  |  |
|  | Al3+, Fe3+, Cr3+, Sc3+, Y3+, La3+, In3+, Ce3+, Pr3+, Nd3+, Sm3+, | 900 |  |  |
| +/- 4 | [Fe(CN)6]4- | 500 |  |  |
|  | Th4+, Zr4+, Ce4+, Sn4+, | 1100 |  |  |

Source: Harris, D.C.; Quantitative Chemical Analysis, Macmillan

There is a much more extensive literature on the SIT parameters, we cannot supply a useful list in this manual. See e.g. Chapter IX in Modelling in Aquatic Chemistry, 1997, I. Grenthe, K. Spahiu, W. Plyasunov, OECD Publications, ISBN 92-64-15569-4 or TDB-2 Guidelines for the extrapolation to Zero ionic strength, 2013, I. Grenthe, F. Mompean, K. Spahiu, H. Wanner, OECD Data Bank.

The following SIT parameters have been used for the Cu/cyclam system. Many values are educated guesses, some are taken from Modelling in Aquatic Chemistry, listed above.

Further required is all information on other ions that do not take part in the equilibria, they are sometimes called spectator ions, here they are referred to as the Counterions and Reactlab™ KINSIM requires their concentrations. This information is listed in each measurement worksheet, see Figure 26. The equations used to define all counterion concentrations are discussed just below Figure 20.

Figure 26: The definition of the spectator ion concentrations in the ‘Measurement’ worksheet(s).

## Note on activity coefficient approximations

All activity approximations are not perfect; the higher the ionic strength the less accurate they are. The Davies approximation handles higher ionic strength solutions better. Pitzer equations are more accurate at higher concentrations but they are complex and require the determination of many additional and specific parameters. These parameters are often not known, and other approximations are used.

The important thing is that an approximation is better than nothing. Additionally, the traditional time-consuming effort for the determination of thermodynamically proper equilibrium constants by extrapolation to zero ionic strength is avoided.

## Testing the results

If ReactLabTM KINSIM is used in the concentration mode, the testing of the results is relatively easy. Some numerical testing is done below, based on the Cu/cyclam system.

The differential d[Cu]/dt can be calculated in two ways; (a) based on kinetic theory:

At any time, this equation has to be fulfilled, the derivative d[Cu]/dt can be calculated numerically in the spreadsheet, it is done in the column O of the worksheet ‘check concentration mode’. The value can be compared, and should be similar to the derivative approximated numerically

Which is done in the column P.

Figure 27: Comparing the differentials d[Cu]/dt from the differential equations and calculated numerically. The difference between the two by the dashed black line with a logarithmic scale on the right.

Figure 27 Displays the results graphically, the green (mathematically correct) and the purple (numerical approximation) are virtually indistinguishable. The difference between the two is the black dashed line with a logarithmic y-axis on the right. The difference is around 10-10. If smaller values for the time difference Δt is used, the numerical approximation will improve. We leave such experiments to the user.

A similar test is done for the protonation equilibria. At any time, the product of the relevant species concentrations has to equal the equilibrium constant. E.g.

These tests are recorded to the right of the kinetic testing, as is obvious the test is passed.

In any of the activity modes the situation is slightly more complex. The equations now are

and

And the resulting Figures are below, note that the x-axis contains the spectrum number rather than the reaction time; the dramatic improvement of the numerical approximation with small Δt is better visible. These results are to be found in the ‘check activities mode’ worksheet

Note, the plots are correct in the supplied spreadsheet for the last selected mode. If the concentration mode was used, the concentration check is correct. If the activity check is performed, the simulation has to be done in any of the activity modes.

## Numerical and other options

A range of numerical calculation and other options are provided in the ‘Main’ sheet that can be adjusted.

Figure 28: The array of numerical option definitions

All numerical approximations are iterative and require the definition of one or several termination criteria. The precision numerical integration of the kinetics derived system of differential equations is defined by an absolute and a relative requirement. There is the option of using a standard Runge-Kutta algorithm which is optimal in most cases or a stiff solver for stiff problems, usually mechanisms with very fast and very slow kinetics steps. The equilibrium computation is controlled by a maximum number of iterations or a convergence tolerance. Small values result in faster computations, large values in more precise results. In rare cases where there appear suspicious results, it is worthwhile increasing the numerical precision by reducing these parameters.

## KW

This is an advanced feature and allows adjustment of the log of the ionic product of water Kw (1.00 x 10-14 M2) if required. logKw is -14 for pure water at room temperature but changes with mixed solvents and temperature. This equilibrium is automatically included when modeling reactions involving protonation equilibria to correctly deal with [H] and therefore pH. See below for more details.

## Handling protonation equilibria, [H], [OH] and Kw

In some cases, reaction models in aqueous solution may include protonation equilibria. In such instances the autoprotolysis of water has to be taken into account. This is done automatically by ReactLab™ if any of the components in the system is identified as ‘H’ which is interpreted as a proton. This is one of the very few rules about the names of components or species in ReactLab™. As a consequence of the presence of protons it is assumed that the reaction is performed in aqueous solution and the equilibrium

is internally added to the present model. Further the hydroxide, OH, will be added automatically to the list of species.

The ionic product Kw is represented as:

The unionized water is omitted from the expression as its concentration is essentially constant. The value of KW is then 1.00x10-14 M2 for pure water at room temperature. If measurements are taken at a different temperature, solvent or ionic strength the correct value for logKw is stored in cell ‘L10’ of the ‘Main’ worksheet.

If protons are involved in a process and the autoprotolysis of water should not be implemented automatically, an easy option is to call it ‘P’ or anything but ‘H’.

## Model entry syntax rules

- A kinetic reaction step is indicated by the symbol ">", e.g.: A+B>C

- The parameter associated with “>” is a rate constant (1st or 2nd order), units are moles and seconds.

- Reversible kinetic steps (slow equilibria) are expressed over two lines i.e. A+B>C C>A+B In this case both forward and reverse rate constants are independently defined and can be fitted.

- A rapid (instantaneous) equilibrium is indicated by "=".

- The parameter associated with “=” is an equilibrium constant K. This can be fitted but the microscopic forward and reverse rates are not accessible.

- The parameter when "=" is used for an equilibrium is log10 of the association equilibrium constant, logK. Note this means it can be negative or positive depending on whether the equilibrium constant itself is less or greater than 1.

- Each equilibrium can only have one product.

- No two equilibria can have the same product.

- Reactants and products within a particular step are combined using the "+" symbol.

- Species names can be single or multiple characters.

- The stoichiometries of species in a particular step are indicated by an integer to the left of the species name (no integer is assumed to mean a stoichiometry of 1).

- The species name ‘H’ is treated as a special case and if present will enable the automatic incorporation of calculations to account for the autoprotolysis of water. In this situation ‘OH’ will also be added to the participating species list.

Note the relationships between logβ’s and logK’s:

logβ1n = logK1 + logK2 + logKn

and

logKn = logβ1n – logβ1n-1

see Maeder and Neuhold for further information.

# PART 2: EXAMPLES

EXAMPLE 2: ENZYME KINETICS (Enzyme_1.xlsx)

Summary:

- A basic enzymatic reaction

- Formation of an activated intermediate, followed by the transformation of the product complex and the release of the product

Figure 29 displays the ‘Main’ worksheet (Enzyme_1.xlsx) which models the reversible interaction of the substrate S with the enzyme E followed by subsequent irreversible product formation.

In the first reversible step the enzyme reacts with the substrate to form the intermediate ES which subsequently forms, in an irreversible step, the enzyme product complex EP which in turn decays, also irreversibly, to release the product P and the enzyme which can start another cycle.

The advantage of ReactLabTM KINSIM is that it can be used to simulate enzyme kinetics under both steady state conditions (see Example 4 below) and to investigate the pre-steady state behaviour when substrate and enzyme first interact. Under steady state conditions high substrate concentration means the enzyme bound intermediates are rapidly saturated and remain approximately constant until the substrate is exhausted. Historically this was the only practical way enzyme reactions could be studied and provided the foundation of the development of the steady state assumption and Michaelis-Menten kinetic analysis quantifying enzyme behaviour in terms of KM and kcat. (Refer to the Michaelis-Menten entry in Wikipedia or any Textbook on Biochemistry for additional information).

In a so-called single-turnover experiment we can ‘observe’ the kinetics during an experiment where the substrate and enzyme concentrations are similar ([E]0 = 10-5 M, 
[S]0 = 2×10-5 M). For the given initial concentrations ([E]0=10-5M, [S]0=2×10-5M) and rate constants (k1 = 106 M-1 s-1 , k-1 = 10 s-1 , k2 = 2 s-1 and k3 = 4 s-1 ) the first fast step is the formation of the ES complex, this phase of the reaction takes about 100 msec. The next phase consists of the transformation of the ES into the EP intermediate which in turn reacts further to release the product. The complete transformation of the substrate takes about 5 sec; at that time the enzyme is also released completely and reaches the initial concentration while the product reaches the initial concentration of the substrate. Note, we use a logarithmic timescale for this simulation, there are many time points at the beginning of the reaction and fewer towards the end where the reactions are slow.

ReactLabTM KINSIM calculates the concentration profiles for the predefined timespan and stores them in the columns B to G; they can be plotted in Excel in the usual way.

Figure 29: The Enzyme_1.xlsx example workbook

It is worthwhile noting that the concentrations seen in the above example are often not observable in traditional absorption spectroscopy. In contrast, fluorescence emission spectroscopy can be much more sensitive and for some substrates the above reactions can be observed.

## EXAMPLE 3: ENZYME KINETICS REVISITED (Enzyme_2.xlsx)

Summary:

- Enzymatic reaction

- Pseudo-first order catalytic reaction

In this example we continue with the same enzymatic mechanism but explore its behavior under different conditions.

The initial concentration of enzyme, E, and substrate, S, were previously chosen to demonstrate the initial interactions to form the first and second intermediates, ES and EP, and the final product, P. This was a so-called single-turnover or pre-steady-state experiment. Such small concentrations for the substrate enable the pre-steady state transient kinetics to be observed and the mechanism dissected but now we will see how the system behaves under more biologically relevant steady state conditions.

In this example we use the same rate constants but have an initial substrate concentration of 0.01 M, the initial enzyme concentration is still 10-5 M. Plotting the concentration of substrate and product reveals the classical pseudo-zeroth order kinetics: a linear decrease of substrate with the concomitant linear increase in the product concentration. This is the result of the limited amount of enzyme which turns over the substrate at a constant rate until it runs out. Under these conditions there is negligible free enzyme during this phase as it is all bound in the form of ES and EP, and the enzyme reaction is proceeding at the maximum rate possible. This is the Vmax in Michaelis-Menten steady state terminology. The rate of continuous product formation during this steady state phase is known as the kcat or turnover number of the enzyme (=Vmax/[E]0) which quantifies how much substrate is converted to product per second.

Figure 30: Pseudo first-order enzyme kinetics, linear decrease and increase of substrate and product

The next Figure reveals some interesting details of the simulation. Part a) only shows the profiles for the enzyme and the two intermediates, ES and EP, (this requires a highly zoomed-in y-axis). Clearly visible is the long time-span where the concentrations of the intermediates are almost perfectly constant; this of course is the well-known steady state situation. Part b) features a logarithmic time scale. The formation of the first intermediate takes about 1 msec after which the free enzyme concentration reaches very low concentration it is almost quantitatively transformed into the first intermediate. After about 1 sec the transformation into the second intermediate has progressed to the steady-state concentrations.

This Figure also demonstrates the ease by which such plots can be created in Excel. We refer to user to many textbooks and help files for Excel.

Figure 31: a) the concentration profiles of the intermediate enzyme complexes demonstrate the steady-state behavior; b) a logarithmic time scale reveals the very fast formation of the first intermediate and transformation into the second intermediate.

## EXAMPLE 4: ENZYME KINETICS REVISITED AGAIN (Enzyme_3.xlsx)

Summary:

- Enzymatic reaction

- Formation of the activated complex as an equilibrium

The initial interaction between the enzyme and the substrate is very fast and normally not observable. It is then not possible to determine the individual values for the forward and back rate constants (k1 and k-1) and it makes sense to define this first step as an instantaneous equilibrium. ReactLab allows the definition of such equilibria very straightforwardly. The ‘=’ sign in the Reaction Type cell defines the reaction as an, instantaneous, equilibrium. Note that equilibrium constants in the parameter cells are always defined as their logarithms to the base of 10. K1 = k1/k-1 = 106 /10 = 105 ; logK1 = 5.

Figure 32: Definition of the interaction between enzyme and substrate as an equilibrium

Comparing Figure 30 and Figure 31with Figure 38 reveals that the concentration profiles are essentially indistinguishable, apart from the first msec where the E+S interaction takes place for the kinetic definition. Here the establishment of the equilibrium concentration of ES is instantaneous.

Figure 38: as Figure 36 with the first step defined as equilibrium

## EXAMPLE 5: THE MICHAELIS-MENTEN ANALYSIS (MichaelisMenten.xlsx)

Summary:

- The Michaelis-Menten way of looking at enzyme kinetics

In order to be able to look at enzyme kinetics in the Michaelis-Menten way we need to simplify the mechanism to:

Using the traditional kcat to represent the irreversible conversion of the ES complex to the product.

The traditional Michaelis-Menten equation states

Where v is the rate of product formation as a given substrate concentration [S]; Vmax is the maximum rate that can be achieved at very high substrate concentration (saturation), KM is the Michaelis constant,

We start by simulating the concentration profiles for a given set of rate constants and initial concentrations.

Figure 33: Rate constants and initial concentrations resulting in the shown concentration profiles

The traditional Michaelis-Menten analysis of an enzymatic reaction involves the determination of the initial reaction rate (at time=0) as a function of the substrate concentration.

Figure 34: Michalis-Menten graph, plotting the reaction rate as a function of the substrate concentration.

We can produce such a graph from the simulation. This can be done by approximating the rate as already demonstrated in Testing the results on page 23

And plot this rate against the substrate concentration [S]. This is done in the Michaelis-Menten workbook

Figure 35: Michaelis-Menten plot for the simulated data (see the Michaelis-Menten worksheet).

This data set can be analysed in the usual way based on the above equation. We use the excel solver for this task: we guess values for Vmax (2e-4) and KM (2e-3) and calculate the corresponding curve which we plot into the above graph:

Figure 36: The Michaelis-Menten curve with not perfect guesses for Vmax  and KM

We can run the excel solver to minimize the sum of squared of the differences between the above curves. For novices, please study the excel manuals and the supplied worksheet for information about using the solver, note also the excel command =SUMXMY2(L9:L56,M9:M56) in cell M4 for the efficient calculation of the sum of squares. The result, with annotations similar to Figure 34 is:

Figure 37: After fitting the parameters; an essentially perfect fit with the fitted parameters: Vmax (0.998e-5) and KM (1.01e-3) which are very close to the known values of 1e-5 and 1e-3 used to generate the data.

Often it is not possible to determine the product concentration directly, it is more common to measure the absorption of the solution, which is the sum of the absorbencies of the S and P. Using random numbers for these molar absorptivities we can generate an absorption trace. The analysis is based on the same equation, only Vmax which now is somewhat arbitrary as it depends on the molar absorptivities KM results again in a perfect fit with KM=1.007e-3.

Figure 38: Using absorbance measurements for the Michaelis-Menten analysis

## EXAMPLE 6: LOTKA-VOLTERRA / PREDATOR-PREY ‘REACTION’ (LotkaVolterra.xlsx)

Summary:

- An interesting biological reaction model

- Perfect ‘oscillating’ reaction

An interesting ‘kinetic’ model is the predator-prey ‘reaction’ developed by Lotka and

Volterra. Imagine an island with a certain population of sheep and wolves. There are several

‘reactions’: in a first ‘reaction’ the sheep breed, relying on grass as their feed. The reaction

is, of course, biologically not quite correct:

The wolves of the island also multiply but they need sheep as source of energy

(Note that ReactLab does not ‘speak’ English and would not understand the reaction

wolf+sheep→2wolves.)

The wolf population is kept under control by wolves dying a natural death.

The ReactLabTM model entry is just as above:

Figure 39: Model entry for the predator-prey ‘reaction’ model

Figure 40 shows the population of sheep and wolves which is perfectly cyclical. The figure also shows the accumulation of dead wolves. The initial ‘concentrations’ for the simulation are [sheep]0=3, [wolf]0=1, there are no dead wolves at time=0.

The figure includes the parameter slide window. The user is welcome to ‘play’ with the ‘rate’ constants for the three ‘reactions’.

Figure 40: The ReactLabTM KINSIM GUI after the simulation, it shows the cyclic population of the sheep and wolves and the accumulation of dead wolves.

It is straightforward to only display the sheep and wolf populations in an Excel graph.

Figure 41: Excel graph of just the sheep and wolf populations, showing perfect cyclic behavior.

While the wolf population is low, the sheep population grows fast, e.g. around 
time = 2. Once the sheep population is large the wolf population grows fast and the sheep population is decimated, e.g. at time = 8. Subsequently, the wolf population is starved and is reduced to small values and the cycle is re-started.

Figure 42: Plotting the wolf against the sheep ‘concentration’, showing the perfect cyclic behavior.

## EXAMPLE 7: THE BELOUSOV-ZHABOTINSKY REACTION (BelousovZhabotinsky.xlsx)

Summary:

- The simulation of a real chemical oscillating reaction

- A very complex reaction mechanism

The Belousov-Zhabotinsky reaction is a famous cyclic reaction. In contrast to the predator-prey model before this reaction is real. However, the reaction mechanism is not completely understood and a simplified version that is usually used to describe the chemical reaction is chemically/stoichiometrically not quite correct.

The Belousov-Zhabotinsky (BZ) reaction involves the oxidation of an organic species such as malonic acid (MA) by an acidified aqueous bromate solution in the presence of a metal ion catalyst such as the Ce(III)/Ce(IV) couple. At excess [MA] the stoichiometry of the net reaction is

A reasonable approximation for the cyclic behavior results from the following reaction mechanism.

With rate constants and initial concentrations as defined in the spreadsheet in Figure 42 the cyclic behavior of Figure 43 results.

Figure 43: The Belousov-Zhabotinsky reaction in ReactLabTM. Model and initial concentrations

The numerical integration delivers the concentration profiles for all species. Figure 43 a) demonstrates the slow conversion of the malonic acid and bromate to HOBr. The oscillations for the remaining species are not visible as these concentrations are too low. Using a logarithmic concentration axis reveals the oscillations, Figure 43 b).

Note that this example, and also the one following, have been integrated numerically using the stiff solver, see NUMERICAL AND OTHER OPTIONS and Figure 24 for more information.

## EXAMPLE 8: THE REACTION OF Fe3+ WITH THIOCYANATE, SCN-, (Fe_SCN.xlsm)

Summary:

- Effects of ionic strength and activity coefficients

This example is based on the work of Prof. Kevin de Berg at the Avondale college in Cooranbong, Australia. Kevin has followed the history of the investigation of the reaction and has also reanalysed the equilibrium as well as kinetics of the interaction using activity-based analysis methods.

The reaction of Fe3+ ions with thiocyanate is a classical chemical reaction. The formation of the red-brown complex can be used as a test for Fe3+ and for SCN-. Here we concentrate on the kinetics of the interactions.

There are two consecutive complexation steps:

And as usual in aqueous solution there are also protonation equilibria that can play a role. Most important here are the protonation of SCN- and the formation of Fe-hydroxo species. The following reactions are most prominent, there are many additional and complex hydroxo species known.

The interference of the hydroxo species are most annoying and its formation can be suppressed by the addition of acid. The protonation of the thiocyanate is only occurring at very low pH, we include this protonation equilibrium for the sake of completeness.

Figure 44: The model entry in the ‘Main’ worksheet.

The equilibrium constants for the two complexation interactions, as well as the constant for the protonation of thiocyanate and the formation of the Fe-hydroxo species are taken from (Kevin de Berg, Marcel Maeder, Sarah Clifford; The thermodynamic formation constants for iron(III) thiocyanate complexes at zero ionic strength; Inorganica Chimica Acta, 466, 2017, 249-253). This leaves the two forward rate constants as unknowns, the back-reaction rate constants are defined via the know equilibrium constants, e.g. k-1=k+1/K1. For the simulations we use values that have been provided by Kevin de Berg, the publication is in progress.

A few words on the initial concentrations of the reactants are required. The molar absorptivities of the Fe(SCN) and Fe(SCN)2 complexes are about 5000 and 10000; thus in order to result in reasonable absorptions for real measurements, the initial Fe concentration should be about 10-4 M, we use[Fe]init = 1.5 × 10-4 M taking into consideration that some of the Fe will remain un-complexed. For significant concentrations of both complexes to form, a large excess of SCN- is required, we choose [SCN]init = 0.25 M. To suppress the formation of Fe(OH) we added 0.1 M acid.

Figure 45: Simulation in ‘concentration’ mode

The result of the simulation in the ‘concentration’ mode is shown in Figure 45. We observe a fast disappearance of the free Fe3+, the formation of the intermediate Fe(SCN) which features a concentration maximum at around 0.1 sec. About half of this complex reacts further to form the Fe(SCN)2 product. There is nothing surprising, so far.

#### The inclusion of activity coefficients

The activity coefficients for ionic species are estimated using different approximative formulas. For more detailed and theoretical information on activities, see Approximations of activity coefficientsError! Reference source not found. on page Error! Bookmark not defined.. All approximations require the knowledge of the total ionic strength of the solution at any time during the reaction. Recall, the equation for the definition of the ionic strength, I, which includes the concentrations of all ions, including the spectator ions:

To be able to compute all ion concentrations, we need to completely understand the composition of the solution.

#### Definition of initial concentrations

The composition of the solution at time = 0 is:

1.5 × 10-4 M Fe(NO3)3 • 9H2O

0.25 M NaSCN

0.1 M HCl

We need to explain how this information translates into the entries in Figure 46:

Figure 46: The definition of all concentration entries for the Fe/SCN simulation

ReactLabTM KINSIM requires the independent definition of the initial concentrations, distinguishing between the equilibrium and the kinetic species. The kinetic species are those what only take part in kinetic reactions, they are the reactions with the forward  arrow. In this example there are two kinetic only species, FeSCN and FeSCN2. Both have 0 initial concentration. Note, the distinction between kinetic and equilibrium species is made by the compiler, not the user. The grey headings in the Figure above are all populated automatically upon ‘Compilation’.

In the example, the equilibrium components are Fe, SCN and H, with the given total concentrations. All other equilibrium species, such as HSCN, FeOH, OH are calculation from the component concentrations and the given equilibrium constants.

What is left is the definition of the concentrations for all spectator ions, they include the nitrate, NO3-, Na+ and Cl- ions. All these concentrations do not change during the reaction. Starting with the [Na+], it is equal to the [SCN] initial concentration. The entry under the (+) cation heading, cell I22, is ‘=J19’ referring to the concentration under the ‘SCN’ heading. Such referencing is not required but makes life easier if we want to ‘play’ with the NaSCN initial concentration, exploring how it affects the formation kinetics.

ReactLabTM KINSIM does not distinguish between the different (-) anions (and all other ions for that matter). Thus, the entry below the (-) anion heading is the sun over all (-) ions, i.e. nitrate and chloride. The entry is ‘=3*I19+K19’ adding up 3 times the initial Fe concentration and the HCl concentration. All other ion concentrations are 0.

Note, it is easy to make a mistake with these counterion definitions. ReactLabTM KINSIM checks for the overall neutrality of the solution and will issue a warning if this is not met. It is only a warning; the user has to be aware that there will be errors with the ionic strength calculations and subsequently the activity coefficient estimations as well.

The different activity approximations require additional information, see Figure 47 for this example.

Figure 47: additional information required for the different activity approximations

The first information are the charges of all species, these are listed in the first ow below the name headings. We deal with Fe3+, SCN-, etc. This is all that is needed for the Debye-Hückel and Davies equations. The extended Debye-Hückel approximation requires additional estimate of the ionic radii, in pm. Such information is not easily found, refer to the Table on page 22, for some values, the remaining values can be estimated. The SIT approximation requires the interaction parameters between all species and the spectator ions. Again, many of these values are known and can be found in e.g. Chapter IX in Modelling in Aquatic Chemistry, 1997, I. Grenthe, K. Spahiu, W. Plyasunov, OECD Publications, ISBN 92-64-15569-4 or TDB-2 Guidelines for the extrapolation to Zero ionic strength, 2013, I. Grenthe, F. Mompean, K. Spahiu, H. Wanner, OECD Data Bank. The values for the interactions of FeSCN and FeSCN2 and FeOH with Na+ have been estimated only.

#### The Debye-Hückel approximation

And now we are ready to simulate the kinetics using any of the activity coefficient approximations, we start with the basic Debye-Hückel equation by selecting .

The immediate result is quite surprising, see Figure 48.

Figure 48: The concentration profiles for the activity-based simulation. Compare with the concentration-based result on the right.

The reactions are much slower! Why is that the case? Let’s consider the first reaction step for which the rate equation is:

We will see in a moment that the ionic strength in the reacting solution is quite high and thus the activity coefficients are low. Thus, the activities are much lower than the concentrations and consequently the rates are also much lower. This is of course is the case for all reactions.

Figure 49: Ionic strength I and all activity coefficients as a function of time.

The left panel of Figure 49 shows the development of the ionic strength I as a function of the reaction time. There is a very small decrease as the Fe3+ ion reacts to form complexes of a lower charge. Remember, the ionic strength is a function of the square of the charge of all ions. HCl and NaSCN are in a substantial excess and thus I only decreases marginally. The right-hand panel displays the activity coefficients of all species. In the Debye-Hückel world all ions of the same absolute charge have the same activity coefficient, e.g. HO-, SCN-, FeSCN2- all have an activity coefficient of about 0.65 at the ionic strength. Ions with  higher charges have lower activity coefficients, e.g. γFe3+  0.02. The activity coefficient of the neutral HSCN is 1.

It is interesting to compare the concentration profiles with the profiles of the activities, see Figure 50. The two sets of profiles are substantially different which is of course the result of activity coefficients substantially different from 1.

Figure 50: Comparing the concentration profiles on the left with the activity profiles on the right (all activity profiles shown with dashed lines).

The worksheet ‘Activities’ contains all activity related information, see Figure 51. Included are the total ionic strength, all activity coefficients, and all activities. Note, as the ionic strength changes during the reaction, the activity coefficients change accordingly. All information can be plotted in the normal way.

Figure 51: All activity related information is collected in a dedicated worksheet, ‘Activities’.

#### The different activity coefficient approximations

It is worthwhile comparing the different approximations for the activity coefficients. The worksheet ‘different approximations’ which is part of the workbook collects all results.

Figure 52: The simulated concentration profiles for the Debye-Hückel, the extended Debye-Hückel, the Davies and the SIT approximations.

The obvious question arises: which approximation is best? It is well beyond the scope of this manual to seriously tackle this question. First: at the high ionic strength of this reaction (I0.35M), all approximations are not reliable. There are significant similarities between the more advanced approximations, compared to the original Debye-Hückel equation. Amongst those, the Davies equation is simplest as it only requires the knowledge of the charges of the ions, the most recent SIT approximation requires the knowledge or the guesses for the interaction of the reacting species with all counter ions existing in the solution. The extended Debye-Hückel requires the ionic radii of all ions. It is up to the user to investigate the validity of the different approximations.

Figure 53: Comparing the activity profiles for the different approximations

Figure 53 compares the activity profiles for the Fe species. As expected, there are similarities between the profiles for the extended Debye-Hückel, the Davies and the SIT approximations. The profiles for the Debye-Hückel equation is substantially different.

Figure 54: Activity coefficients for Debye-Hückel (), extended Debye-Hückel (), the Davies () and the SIT () approximations  for different charges. The vertical black line indicates the ionic strength at the simulations.

Figure 54 displays the activity coefficients for the different approximation equations. Interesting observations are that the activity coefficients can increase at high I for the Davies and SIT approximations. The Debye-Hückel values are consistently the lowest, which explains the difference in the results, as shown in Figure 52 and Figure 53.

## EXAMPLE 9: THE REACTION OF Fe3+ WITH 3 LIGANDS L, LARGE CHANGES IN IONIC STRENGTH (Fe_L3.xlsm)

Summary:

- More on substantial changes in ionic strength

The conditions in the example 8, the reaction of Fe3+ with thiocyanate are close to realistic, a considerable excess of a strong acid was added to prevent the formation of FeOH species and a large excess of SCN- was used to result in reasonable formation of the complexes. All this resulted in an excess of non-reacting ions and thus virtually constant ionic strength. In some ways the conditions mimic the old-fashioned approach of enforcing constant ionic strength through the addition of inert salts.

In a similar reaction we mimic a situation where the ionic strength changes substantially, this is done by removing the formation of the hydroxy species and thus the necessity of adding the acid. Further, the complexation equilibria are much more on the product side, no longer necessitating and excess in ligand. We also added the formation of the 1:3 complex.

The new model is:

3 reversible reactions between Fe and a ligand L, the forward rate constants are 130, 100, and 50 M-1sec-1 and the equilibrium constants, defining the back-rate constants are 10000, 5000, and 3000 M-1sec-1.

The initial concentrations are, [Fe3+]init = 0.1 M, [L-]init = 0.35 M; the spectator ions are calculated in the usual way:

The other activity parameters, such as ionic radii and interaction parameters are left as in the Fe/SCN example.

Figure 55: the concentration profiles for the fictive Fe/L system

The observations are similar to the previously for the Fe/SCN system: the reactions appear fastest with no activity correction, slowest for the Debye-Hückel approximation and similar for the other approximations.

Figure 56: There is a substantial change in the ionic strength and concomitant in the activity coefficients

The main difference is that now the ionic strength drops substantially from a value close to 1 down to about 0.4. Concomitantly the activity coefficients increase substantially during the reaction.

Several questions arise: what is gained by the inclusion of activity coefficients? The activity estimations are not perfect, how far can we trust them? Is it not much more convenient to ignore all that?

Here an attempt at a reply: Indeed, the estimations for the activity coefficients are not perfect, they are just estimations, However, assuming all activity coefficients to be one (‘concentration’ mode) or constant (assuming constant ionic strength) are also not perfect, Indeed, they are almost always further from the truth than imperfect activity approximations. There is no need to ignore the changes in activities as we have now a tool which allows the straightforward simulation of approximately correct reaction kinetics.

Note: the ionic strength will not ever reach 0 in reactions of this kind, even if the transformation to the neutral FeL3 would be complete. There are always spectator ions around, in the example the anions for the Fe salt as well as the cations for the cyanate salt.

One last adaptation of this example. What happens when we decrease the total  concentrations and thus the effects of the ionic strength? Reduction of the initial concentrations by a factor of 100 results in:

The ionic strength is always low, activity coefficients much closer to 1 and all concentration profiles are similar. If such conditions can be maintained, it may be tempting to ignore activities. However, there is no need and the activity coefficient are not 1 nor are they constant. Indeed, in such examples the approximations are very good.

## References

Marcel Maeder, Yorck-Michael Neuhold. Practical Data Analysis in Chemistry, Elsevier 2007

John Ross, Igor Schreiber, Marcel O. Vlad, and Adam Arkin. Determination of Complex Reaction Mechanisms: Analysis of Chemical, Biological, and Genetic Networks. Oxford University Press 2005

Robert W. Hay. Reaction Mechanisms of Metal Complexes. Albion/Harwood Pub 2000

H. Gutfreund. Kinetics for the Life Sciences. Cambridge University Press 1995

James H Espenson. Chemical Kinetics and Reaction Mechanisms (2nd edition). McGraw-Hill 1995 S.K. Scott. Oscillations, Waves, and Chaos in Chemical Kinetics. Oxford. Oxford Chemistry Press 1994.

Ralph G. Wilkins. Kinetics and Mechanisms of Reactions of Transition Metal Complexes. VCH 1991

A. Fersht. Enzyme Structure and Mechanism. Freeman 1985 S.W. Benson. The Foundations of Chemical Kinetics. McGraw-Hill New York 1960

Arthur Martell, Robert Hancock. Metal Complexes in Aqueous Solutions (Modern Inorganic Chemistry). Springer 1996

I. Grenthe, K. Spahiu, W. Plyasunov, Chapter IX in Modelling in Aquatic Chemistry, 1997, OECD Publications, ISBN 92-64-15569-4

I. Grenthe, F. Mompean, K. Spahiu, H. Wanner, TDB-2 Guidelines for the extrapolation to Zero ionic strength, 2013, OECD Data Bank.